










File: List&Checklists
Winter Storage Prep / Inside garage - Car      

1.  
Change and top off all fluids as necessary.  Engine oil less than 1000 mileage is OK. 
              Assuming 100% synthetic is used.

2.
Use fuel stabilizer  (Sta-bil (TM)  Full tank.  See Sta-bil instructions.

3.
Prep, wash and/or wax exterior, clean/vacuum interior.

4.
Apply silicone spray lubricant to door and trunk seals.

5.
Check battery & top off cells with distilled water.

6.
Lay out plastic tarp (floor covering).  This helps to prevent moisture accumulation.

7.
42 PSI in tires (for 35 PSI normal).  (use the home mini pump, start pump B4 attaching 
              to tire stem).

8.
Support wiper blade arms with rags (2).

9.
Transmission in REVERSE.

10.
Emergency brake - OFF.

11.
Wheel Chocks (2)  ON.

12.
Ventilator NOT set to 're-circulate'.

13.
Windows HALFWAY DOWN.

14.
 Remove any CDs.

15.
 California Car Cover.
16.
 Battery Tender.

17.
 Contact Insurance company on non-use storage.


